Electrical Installations and loads

Task 1: Understanding wires and protective devices

Using the wires provided, please fill in the following table:

| Wire size (mm2) | Number of strands (please count) | Diameter of one strand (mm) | Total area (mm2) | Resistance per meter length(10-3m-1) |
| --- | --- | --- | --- | --- |
| 1 | 1 | 1.13 | 1.0029 | 16.7514 |
| 1.5 | 7 | 0.53 | 1.5443 | 10.8787 |
| 2.5 | 7 | 0.67 | 2.4680 | 6.8071 |
| 4 | 7 | 0.85 | 3.9722 | 4.2294 |

Please note that the resistivity of Cu is 16.8 nΩ·m

Task 2: Understanding the protective devices

Using the following-coloured parts of the CB, write down the operation of each part:

|  | Latch – mechanically holds the contacts closed. Allowing current to flow. When tripped latch releases, causing contact to separate Solenoid- Solenoid creates a big magnetic field when a large current passes through it, which move plunger and move the latch, which open the circuit then. |
| --- | --- |
|  | Made of two distinct metals, it heats up when current flowing through it. It designed in a way that after a certain predefined current it bends such that the latch will trigger, which cause to open the circuit. |
|  | When the MCB contacts released, an electrical arc can form. This arc chutes is designed to dissipate this arc safely. |

Write ‘True’ or ‘False’ under each of the following statements related to an MCB (Miniature Circuit Breaker) or MCCB (Moulded Case Circuit Breaker):

| The circuit breaker should continuously carry the normal load current | True |
| --- | --- |
| The circuit breaker must detect fault conditions and promptly interrupt the fault current | True |
| The circuit breaker should not be damaged during the arc quenching process that occurs when interrupting fault currents | True |
| After a fault is cleared, an automatic or manual mechanism should be available to restore the circuit's operation | True |

Perform the following calculation for a single-phase circuit powering a 5 kW oven:

| Possible voltage to which the over is connected | 230V |
| --- | --- |
| Possible power factor (cosϕ) of the oven. Please remember that an over carries a resistive element and current and voltage waveforms are in-phase. That is ϕ=0 | 1 |
| Using the equation P=VIcosϕ, calculate the current drawn by the oven | 21.74A |
| Using available MCB sizes (look on the internet) select an MCB such that the MCB rating is greater than the load current | 32A |

- Describe the operation of an RCCB with a diagram

An RCCB works by comparing the current in the phase and neutral wires, which pass through a common sensing coil. Under normal conditions, both currents are equal, so their magnetic fields cancel out. If a leakage occurs (like current flowing to the ground through a person), the balance breaks. This imbalance induces a current in the sensing coil, triggering the relay. The relay then trips the breaker, cutting power—usually in under 30 ms—preventing electric shocks and hazards.

Task 3: Wiring System

In this experiment, you will utilize the existing electrical installation to connect the circuits depicted in the following figure.

  - Circuit 1: A radial lamp circuit.

  - Circuit 2: A radial socket outlet circuit.

  - Circuit 3: A ring socket outlet circuit.

- Based on the details provided in the diagram, your observation of the laboratory setup components, and verification of the provided wiring, connect both circuits.

- Present these completed circuits to the instructor for inspection.

- Test the functionality of the connected circuits by connecting the different appliances given abd measuring the current.

- Subsequently, disconnect the main power supply to the distribution board (DB).

Task 4: Design the Electrical System

Objective: To apply electrical wiring principles to design a safe and functional electrical system for a residential building.

Material: Floor plans

Activity Steps:

Step 1: List the appliances/items in each room

| Room | Description of electrical items in each room (lamps, fans, socket outlets, special appliances) |
| --- | --- |

Step 2: Implementing the plan with electrical items

Implement the plan assigned to your group in https://floorplanner.com/projects

- Implement the plan on the ‘floorplanner. ’

- Plan the layout of electrical outlets, switches, and lighting fixtures on the floor plan.

- Using the Place Sign and Symbols tab, indicate the locations of switches, plug sockets, lamps, and fans.

Step 3: Develop an Excel sheet with the following subheadings

For each sub-circuit that you designed, develop an Excel sheet with the following subheadings:

- Circuit No ✅

- Ring/ Radial ✅

- Description ✅

- Diversity factor ✅

- Maximum demand (A) ✅

- Rating of the MCB (A) ✅

- Length of the circuit (m) ✅

- Min Current Carrying Capacity ✅

- Ambient temperature factor ✅

- Grouping factor ✅

- Initial wire selection (mm2) ✅

- Voltage drop	✅

- Check whether the  VD < allowable value✅

- If required,	Second wire selection & Voltage drop

Propagate the Excel sheet for each circuit

Step 4:  Create a detailed wiring diagram showing the connections between the electrical components and the distribution panel.

DATA:

TABLE A: Diversity factors

| Final circuit | Household installations | Small shops, stores, offices and business premises | Small hotels. Boarding houses, guest houses |
| --- | --- | --- | --- |
| Lightning | 66% of TD | 90% of TD | 75% of TD |
| Cooking appliances | If FL < 10 A 10 A + 5A if a SO is incorporated If FL > 10 A 10 A + 30% of FL + 5A if a SO is incorporated | X + 0.8Y+0.6Z | X + 0.8Y+0.6Z |
| Motors | N/A | X + 0.8Y+0.6Z | X + 0.8Y+0.6Z |
| Instantaneous water heaters | X+Y+0.25Z | X+Y+0.25Z | X+Y+0.25Z |
| Thermostatically controlled water heaters | 100% of FL | 100% of FL | 100% of FL |
| Standard or final circuits with SOs | X+0.4(Y+Z) | X+0.5(Y+Z) | X+0.5(Y+Z) |
| SO other than Standard circuits | X+0.4(Y+Z) | X+0.7(Y+Z) | X+0.7W+0.4Z |

where TD is the total current demand; FL is the full load; SO is the socket outlet; X is the largest appliance/circuit; Y is the second largest; Z is the remaining; W is the SOs in living and dining rooms

TABLE B: Final circuits using square socket-outlets

| Type of circuit | Over current protective device | Minimum live conductor cross sectional area (mm2) | Maximum floor area served (m2) |
| --- | --- | --- | --- |
| Ring | 30 or 32 A | 2.5 | 100 |
| Radial | 30 or 32 A | 4.0 | 75 |
| Radial | 20 | 2.5 | 50 |

TABLE C: Current carrying capacities and associated voltage drops for single-core p.v.c insulated cables, non-armoured, copper conductors enclosed in conduit or trunking:

| Cross sectional area (mm2) | 2 cables single phase ac or dc |  | 3 or 4 cables three phase ac |  |
| --- | --- | --- | --- | --- |
|  | Current (A) | Drop in mV/Amp-meter | Current (A) | Drop in mV/Amp-meter |
| 1.0 | 13.5 | 44.0 | 12.0 | 38.0 |
| 1.5 | 17.5 | 29.0 | 15.5 | 25.0 |
| 2.5 | 24.0 | 18.0 | 21.0 | 15.0 |
| 4.0 | 32.0 | 11.0 | 28.0 | 9.5 |
| 6.0 | 41.0 | 7.3 | 36.0 | 6.4 |
| 10.0 | 57.0 | 4.4 | 50.0 | 3.8 |
| 16.0 | 76.0 | 2.8 | 68.0 | 2.4 |
| 25.0 | 101.0 | 1.8 | 89.0 | 1.5 |
| 35.0 | 125.0 | 1.3 | 110.0 | 1.1 |

Ambient temperature 300 C

TABLE D: Ambient temperature factor

| Ambient temperature oC | 25 | 30 | 35 | 40 | 45 | 50 | 55 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Factor | 1.03 | 1 | 0.94 | 0.87 | 0.79 | 0.71 | 0.61 | 0.50 |

TABLE E: Grouping factor

| Number of circuits | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Factor | 1.0 | 0.8 | 0.7 | 0.65 | 0.6 | 0.57 | 0.54 | 0.52 |

Step 1: Description about each part of the house

Description about each part of the house

Living Room
Six 10W LED lamps
Two 10W wall lamps
Four 13 A plug sockets 
One 75 W ceiling fan

Dining and Pantry
Two 5W LED lamps
Two 10W LED lamps
One 15W LED lamp
Four 13 A plug sockets
One 75 W ceiling fan
One 2kW Oven
One 2kW Rice Cooker
150W Fridge

Bedroom 1
One 10 W LED lamps 
One 13 A plug sockets
One 75W Fan

Bedroom 2
One 10 W LED lamps 
One 13 A plug sockets
One 75W Fan

Bedroom 3
Two 10 W LED lamps 
Two 13 A plug sockets
Two 5W LED lamps
One 1.2kW A/C

Toilet
One 10W LED lamp
One 5W LED lamp
One 1 kW instantaneous water heater
Two 13A plug sockets

Garage
Two 10W LED lamp
One 15W LED lamp
Two 13A plug sockets

Step 2: The plan with electrical items

| Circuit No | Ring / Radial | Description |
| --- | --- | --- |
| Lighting |  |  |
| 1 | Radial | - One 10 W LED lamps in bedroom 1 - One 10 W LED lamps in bedroom 2 - Two 10W and 5W LED lamps in bedroom 3 |
| 2 | Radial | - Two 5W LED lamps in the dining & pantry - Two 10W LED lights in dining & pantry - One 15W LED lamp in the dining & pantry |
| 3 | Radial | - One 10W LED and One 5W LED in Toilet - Two 10W LED and 5W wall lamp in the Corridor |
| 4 | Radial | - Six 10W LED lamps and two 5W wall lamps |
| 5 | Radial | - Two 10W LED lamps and one 15W lamps |
| Socket Outlets |  |  |
| 6 | Ring | - One 13A socket in bedroom 1 - One 13A socket in bedroom 2 - Two 13A sockets in bedroom 3 - Two 13A sockets in toilet |
| 7 | Radial | - Four 13A sockets for dining & pantry - Four 13A sockets in living room - Two 13A sockets in garage |
| 8 | Radial | - A dedicated socket for 150W fridge in the kitchen |
| 9 | Radial | - A dedicated socket for 2kW oven in the kitchen |
| 10 | Radial | - A dedicated socket for 2kW rice cooker in the kitchen |
| 11 | Radial | - A dedicated socket for 1kW instantaneous water heater in the toilet |
| 12 | Radial | - A dedicated socket for 1.2kW A/C in the bedroom 3 |
| Ceiling fan circuits |  |  |
| 13 | Radial | - One 75 W ceiling fan in living room - One 75 W ceiling fan in dining & pantry - One 75 W ceiling fan in bedroom 1 - One 75 W ceiling fan in bedroom 2 |

Step 3: Wiring calculations

1. Description of circuit sub-divisions

| Circuit Number | Calculation considering the Diversity factors | Max current demand/A | MCB rating/A |
| --- | --- | --- | --- |
| 1 | 0.66 4×20 W+ 2×5 W230 V | 0.26 | 6 |
| 2 | 0.66 1×15 W+ 4×10 W+2×5 W230 V | 0.19 | 6 |
| 3 | 0.66 3×10 W+2×5 W230 V | 0.11 | 6 |
| 4 | 0.666×10 W+2×10 W230 V | 0.23 | 6 |
| 5 | 0.66 1×15 W+ 2×10 W230 V | 0.10 | 6 |
| 6 | - | - | 32 |
| 7 | - | - | 32 |
| 8 | 150 W230 V | 0.65 | 6 |
| 9 | 2000W230 V | 8.70 | 10 |
| 10 | 2000 W230 V | 8.70 | 10 |
| 11 | 1000 W230 V | 4.35 | 10 |
| 12 | 1200 W230 V | 5.22 | 6 |
| 13 | 4×75 W 230 V | 1.30 | 6 |

2. Diversity factors, Max current and MCB rating demand in each circuit

3. RCCB Selection

| RCCB | Assigned circuits to the RCCB |
| --- | --- |
| 40A, 30mA | 1 |
|  | 2 |
|  | 3 |
|  | 4 |
|  | 5 |
|  | 6 |
| 40A, 30mA | 9 |
|  | 10 |
|  | 11 |
|  | 12 |
| 40A, 30mA | 7 |
|  | 8 |
|  | 13 |

4. Wire Sizes

CCC> MCB RatingCt×Cg

| Circuit Number | Number of circuits in the conduit | Cg | Minimum CCC / A | Maximum wire length/m | Voltage drop / V | L and N Conductor size (mm2) | Protective conductor size (mm2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 0.7 | 60.94×0.7=9.119 | 25 | 44×10-3×25×0.26=0.28 | 1.0 | - |
| 2 | 2 | 0.8 | 60.94×0.8=7.979 | 9 | 44×10-3×9×0.19=0.07 | 1.0 | - |
| 3 | 2 | 0.8 | 60.94×0.8=7.979 | 11 | 44×10-3×11×0.11=0.06 | 1.0 | - |
| 4 | 2 | 0.8 | 60.94×0.8=7.979 | 15 | 44×10-3×15×0.23=0.15 | 1.0 | - |
| 5 | 2 | 0.8 | 60.94×0.8=7.979 | 10 | 44×10-3×10×0.10=0.04 | 1.0 | - |
| 6 | 3 | 0.7 | - | 21 | - | 2.5 | 2.5 |
| 7 | 2 | 0.8 | - | 27 | - | 2.5 | 2.5 |
| 8 | 3 | 0.7 | 60.94×0.7=9.119 | 4 | 44×10-3×4×0.65=0.11 | 1.0 | 2.5 |
| 9 | 3 | 0.7 | 100.94×0.7=15.198 | 5 | 29×10-3×5×8.70=1.26 | 1.5 | 2.5 |
| 10 | 3 | 0.7 | 100.94×0.7=15.198 | 4 | 29×10-3×4×8.70=1.01 | 1.5 | 2.5 |
| 11 | 2 | 0.8 | 60.94×0.8=7.979 | 4 | 44×10-3×4×4.35=0.77 | 1.0 | 2.5 |
| 12 | 3 | 0.7 | 60.94×0.7=9.119 | 5 | 44×10-3×5×5.22=1.15 | 1.0 | 2.5 |
| 13 | 2 | 0.8 | 60.94×0.8=7.979 | 25 | 44×10-3×25×1.30=1.43 | 1.0 | 2.5 |

- Allowable voltage drop is 3%. Therefore, allowable voltage drop   = 230 V ×0.03
							              = 6.9 V

- Ambient temperature factor was taken as 0.94 at 35 oC

Step 4: Wiring diagrams

Lamp layout with switches

Sockets outlet layout

Dedicated circuit

Ring circuit

Radial circuit

Plug socket

LED lamp

Switches

Ceiling Fan

Distribution Box

Fan layout

Conduit layouts

Conduit layout for lights

¾” Conduit

1” Conduit

Conduit layout for sockets outlet

Final conduit layout

RCCB 
40 A

1

L1

L2

L3

L4

L5

6-way MCB

L7

1/1.13

L6

S1

S2

S3

S5

S6

S7

S8

F1

F2

F3

12

1/1.13

S13

S14

S9

S15

S16

S17

S18

S10

S11

S4

S12

4

7/0.67

5

6

7/0.67

7/0.67

3

7/0.67

6A

6A

16A

32A

16A

6A

9. Diagrams

RCCB 
40 A

Conduit layout for sockets outlet

1

2

3

Lighting in dining & pantry

Lighting in bedroom 1, bedroom 2, bedroom 3

L and N- 1 mm2

L and N- 2.5 mm2

40A, 30mA RCCB

Lighting in toilet and corridor

L and N- 2.5 mm2

4

Lighting in living room

L and N- 2.5 mm2

5

Lighting in garage

L and N- 2.5 mm2

6

Plug sockets in bedroom1, bedroom2, bedroom3 and toilet

L and N- 2.5 mm2, E – 2.5 mm2

40A, 1P

L and N- 1.5 mm2, E – 2.5 mm2

10

9

A dedicated line for the 2kW oven in dining & pantry

L and N- 1.5 mm2, E -2.5 mm2

11

40A, 30mA RCCB

A dedicated line for the 2kW rice cooker in dining & pantry

A dedicated line for the 1kW water heater in toilet

L and N- 1.0 mm2, E - 2.5 mm2

12

A dedicated line for the 1.2kW Air conditioner in bedroom 3

L and N- 1.0 mm2, E – 2.5 mm2

7

Plug sockets in dining & pantry, living room, and garage

L and N- 2.5 mm2, E – 2.5 mm2

8

A dedicated line for the 150W fridge in dining & pantry

L and N- 1.0 mm2, E -2.5 mm2

13

40A, 30mA RCCB

Fans in the bedroom 1, bedroom 2, living room, dining & pantry

L and N- 1.0 mm2, E - 2.5 mm2

Approximate Costs (in rupees)

*Note that the costs are given in LKR

| 1 mm2 wire (100 m) | Rs. 5000 – single core | Ceiling roses | Rs. 275 |
| --- | --- | --- | --- |
| 1.5 mm2 wire (100 m) | Rs. 20000 – two core | 13A socket | Rs. 900 |
| 2.5 mm2 wire (100 m) | Rs. 30000 – two core | Switches | Rs. 300 per gang |
| Earth wires (100 m) | Rs. 15000 | MCB | Rs. 800 |
| Fan | Rs.13000 | Conduits 1” | Rs. 750 per m |
| Main switch | Rs. 2500 | Conduits 3/4” | Rs. 500 per m |
| RCCB | Rs. 4800 | Holder | Rs. 225 |
| Bends, Plastic boxes, junction boxes | Rs. 100 | LED lamp | Rs.2000 – 5W wall lamp Rs.1000 – 5W Rs.1500 – 10W Rs.2000 – 15W |

Workmanship fees – Rs.750 per point

Bill Of Quantities (BOQ)

- For ring plug circuit

|  | For circuit 5 (15m) |
| --- | --- |
| For plug sockets | 5400 |
| Cost for L and N wires | 9000 |
| Cost for Earth wires | 2250 |
| Cost for conduit (Took 60% of wire length) | 6750 |
| Cost for bends, plastic boxes, junction boxes | 300 |
| Workmanship fees | 750x6 = 4500 |
| Total | 28200 |

Total cost for ring plug circuits = 28200

- For radial plug circuits

|  | For circuit 10 (5m) | For circuit 4 (12m) |
| --- | --- | --- |
| For plug sockets | 900 | 4500 |
| Cost for L and N wires | 500 | 7200 |
| Cost for Earth wires | 750 | 1800 |
| Cost for conduit (Took 60% of wire length) | 2250 | 5400 |
| Cost for bends, plastic boxes, junction boxes | 100 | 200 |
| Workmanship fees | 750 | 3750 |
| Total | 5250 | 22850 |

Average cost = 14050

Total cost for radial plug circuits = 14050 x 8 = 112400

- For lightening circuits

|  | For circuit 1 (20m) | For circuit 2 (30m) |
| --- | --- | --- |
| For lamps | 15800 | 44700 |
| Cost for L and N wires | 2000 | 3000 |
| Holders | 1575 | 4950 |
| Switches and sunk boxes | 1600 | 1200 |
| Ceiling roses | 1925 | 3025 |
| Cost for conduit (Took 60% of wire length) | 6000 | 9000 |
| Cost for bends, plastic boxes, junction boxes | 700 | 1400 |
| Workmanship fees | 750x7 = 5250 | 750x22 = 16500 |
| Total | 34850 | 83775 |

Total cost for lightening circuits = 118625

- For Fan circuit

|  | For circuit 12 (22m) |
| --- | --- |
| For Fans | 13000 x 3 = 39000 |
| Cost for L and N wires | 2200 |
| Cost for Earth wires | 1000 |
| Switches and sunk boxes | 1200 |
| Cost for conduit (Took 60% of wire length) | 6600 |
| Cost for bends, plastic boxes, junction boxes | 400 |
| Workmanship fees | 750x3 = 2250 |
| Total | 52650 |

Total cost for fan circuits = 52650

Total cost

| Description | Cost (rupees) |
| --- | --- |
| Main switch | 2500 |
| Trip switch | 5000 |
| MCB’s | 9600 |
| Radial plug socket circuits | 112400 |
| Ring plug socket circuit | 28200 |
| Lightening circuits | 118625 |
| Fan circuit | 52650 |
| Total Cost | 328975 |

Total cost = 328,975 LKR